# Standard Operating Procedure (SOP)

Product: Instant Gajar Halwa Premix (Type B)

## Purpose:

To describe the step-by-step process for the preparation, drying, and packaging of Carrot Halwa Premix using standardized ingredients and operating conditions to ensure consistent quality, safety, and sensory characteristics.

## 2. Objective

To develop a uniform method for producing Gajar Halwa Premix that retains authentic flavor, aroma, and nutritional value while being shelf-stable for extended storage.

## 3. Scope

This procedure applies to small-scale or pilot-scale preparation of Gajar Halwa Premix in the Food Processing Laboratory or semi-commercial unit.

## 4. Materials and Ingredients

| Sr. No. | Ingredient | Specification | Quantity (for 1 kg grated carrots) |
| --- | --- | --- | --- |
| 1 | Fresh Carrots | Tender, bright orange, clean, free from damage | 1 kg |
| 2 | Ghee | Pure cow ghee | 20 g |
| 3 | Milk | Fresh cow milk (pre-boiled) | 750 ml |
| 4 | Cardamom powder | Finely ground, aromatic | 2.6 g |
| 5 | Dry fruits (cashew, almond, raisin) | Chopped, clean | As required |

## 5. Equipment Required

Stainless steel karai, Gas stove or induction heater, Stainless steel ladle, Food-grade trays, Hot air dryer, Digital thermometer, Weighing balance, Grater, Measuring cylinders, Packaging materials, Sealing machine, Protective wear.

## 6. Safety Precautions

Ensure clean and sanitized equipment, personal hygiene, careful handling of hot equipment, proper ventilation, and verified dryer calibration.

## 7. Procedure

### Step 1: Preparation of Raw Material

Select fresh, tender carrots with uniform size and color.

Wash thoroughly under running tap water and rinse with potable water.

Peel lightly (optional) and grate uniformly. Discard top green portion.

Weigh grated carrots (1 kg).

### Step 2: Roasting in Ghee

Place karai on medium flame and add 20 g ghee.

Add grated carrots and sauté for 12 minutes with stirring every 2–3 minutes.

Cook until carrots soften and turn glossy.

### Step 3: Milk Addition and Cooking

Pre-boil milk and add 750 ml to roasted carrots.

Cook on medium flame, stirring every 2–3 minutes.

Continue until milk evaporates and mixture thickens (20–25 minutes).

### Step 4: Flavor Incorporation

Turn off flame and add 2.6 g cardamom powder.

Mix thoroughly and cool for 5–10 minutes.

### Step 5: Drying Process

Grease trays with ghee and spread halwa evenly (0.5–1 cm thick).

Dry at 57°C for 7–9 hours until moisture <6%.

Check intermittently for uniform drying.

### Step 6: Cooling and Mixing

Cool product to room temperature (25–30°C).

Add chopped dry fruits and mix uniformly.

### Step 7: Packaging and Labeling

Pack in moisture-proof laminated pouches or vacuum-sealed bags.

Seal and label with product details, batch number, and date.

## 8. Cost Analysis (for 1 kg batch)

| Component | Quantity / Duration | Cost (₹) |
| --- | --- | --- |
| Carrot | 1 kg | 40 |
| Milk | 750 ml | 30 |
| Ghee | 20 g | 16 |
| Cardamom | 2.6 g | 10 |
| Dry Fruits | 40 g | 40 |
| Gas | 30 mins | 7.5 |
| Electricity | 1 tray | 9 |
| Labour | 25% | 50 |
| Packaging (3 pouches) | 3 pouch | 13.5 |
| Total |  | 206 |

## 9. Quality Control Parameters

| Parameter | Standard | Method |
| --- | --- | --- |
| Appearance | Uniform orange color, no burnt particles | Visual |
| Aroma | Pleasant, sweet, and characteristic of carrot halwa | Sensory evaluation |
| Moisture | ≤ 6% | Moisture analyzer / oven dry method |
| Texture | Dry, non-sticky, easy to rehydrate | Sensory check |
| Microbial load | Within permissible limits | Standard plate count |

## 10. Rehydration (Consumer Use)

Add 150 ml of water in a clean bowl and bring it to a boil.
Pour the contents of one Carrot Halwa Premix packet into the boiling water.
Cook on medium flame for 5–7 minutes, stirring occasionally to prevent sticking.
After cooking, add sugar or jaggery as per taste, mix well, and serve hot.

## 11. Notes

Avoid drying beyond 60°C to prevent flavor or color loss. Use fresh milk and ghee. Maintain hygiene to avoid contamination.